Levee Survey Data Summary

Enclosed is a copy of the levee survey data collected in August 2009. Both Edgerly Island and Ingersoll Tract were surveyed. Edgerly had been previously surveyed while Ingersoll Tract had not.

This information is being provided for informational purposes only. Please read the data sheet carefully. If this explanation is not clear please call  the office at 707-255-2996 and we will help interpret the data.

Following is a summary and explanation of the data attached.

Edgerly Island

There are 59 locations (called points in the report) which are approximately every 100 feet or every other lot. There are three types of levee height markers which indicate the elevation at a particular spot.

- Original large eyebolts.  These were installed in 1984 and were a rough indication of a 10 foot elevation.  We were not able to find all of the original markers so more were added.

- Newly installed eyebolts.  These are a little smaller than the original eyebolts and were installed at ground level.

- Permanent Structure points such as concrete or block walls.

When you look at the enclosed data sheet you will see several columns of information.  These columns are explained as follows:

HOUSE NUMBER: This is the street address on Milton Road. If your address is shown 	you should have a marker, either an original eye bolt, a new eye bolt or a permanent 	structure  as used. If your house number is not shown there should be a marker on your 	neighbors lot.

POINT NUMBER: This is the marker identification number. The numbers start at the 	south end of the road with point number 201 located at 1998 Milton and run up river to 	point number 261 at 1500 Milton.

ELEVATION: This number is the actual height, in feet, above Mean Sea Level at the 	TOP of the eyebolt (or concrete wall). The base line for this data is NGVD 1929.

*NOTE: When you look at this elevation number you must also look at the levee itself in 	relationship to the eyebolt. i.e. If the eyebolt is above the ground (levee) level you must 	subtract the distance between the top of the eyebolt and the levee to know the true levee 	height. For example if the elevation is 9.5 feet and the levee is six inches below the top of 	the eyebolt then the levee is really only 9 feet above Mean Sea Level (see "Description" 	below).

The same is true of a concrete wall elevation. If the wall is not the same height for the full 	width of the lot then you must look at the lowest spot on the wall.

DESCRIPTION: This column describes the type and approximate location of the 	marker. Further it also indicates the relative height of the marker above ground, if that is 	in fact the case.

Type of marker;  "Found" or "Set"

If the marker is described as "Found" it means that this is an original eyebolt. If the 	marker is described as "Set" it means that a new eyebolt was installed. All of the eyebolts 	that were "Set" were installed at ground level.

Height above ground: If there is a notation that says "2.5' above ground" (as an 	example) that means that the top of the eyebolt is 2.5 feet above the ground level.  That 	means that the actual height of the ground at the marker is 2.5 feet lower than the 	elevation number. Example: if the elevation is 9.5 feet and there is a notation that says         	+ 2.5'above ground then the actual height of the ground is 7.0 feet above Mean Sea 	Level.

*Note:  If there is a levee wall or structure adjacent to the marker the height of that wall relative to the height of the levee marker will determine the actual height of the levee.

Ingersoll Tract:

There were 19 locations surveyed on Ingersoll Tract (approximately every 100 feet or every other lot). There were no existing (original) markers so all the locations are either new eyebolts (ground level) or permanent structure (concrete wall) therefore all elevations listed are actual heights.

NOTE:  Due to the methods used to survey the levee in 1984 and 1994 there is no way to accurately compare the data acquired in 2009 with the previously acquired data.  In other words, it is not possible to know exactly how much the levee may have settled over time.  However, one can safely assume the levee has settled as all levees do.

If you have any questions, please call the district office at 707-255-2996.

PLEASE RETAIN THIS WITH YOUR OTHER HOUSEHOLD DOCUMENTS

| HOUSE NO | POINT NO | ELEV | DESCRIPTION |
| --- | --- | --- | --- |
| 1998 | 201 | 9.25' | Found eye bolt, south end, ± 10' north of north edge of conc boat ramp |
| 1998 | 202 | 7.04' | Edge of conc at top boat ramp |
| 1998 | 203 | 8.98' | Set eye-bolt, north end ± 8.5' south of conc wall |
| 1920 | 204 | 9.34' | Found eye-bolt, south end, near chain link fence, ± 2.6' above ground |
| 1916 | 205 | 9.31' | Found eye-bolt, north end, near chain link fence corner ± 2.5' above ground |
| 1904 | 206 | 9.02' | Found eye-bolt, south end, at concrete wall ± 2.2' above ground |
| 1900 | 207 | 8.70' | Found eye-bolt, north end |
|  | 208 | 8.65' | Set eye-bolt, NRD lot north of 1900, ± 4' south of conc wall |
| 1888 | 209 | 7.91' | Top of wall, south end of wall |
| 1884 | 210 | 8.39' | Top of conc wall, north end of wall |
| 1876 | 211 | 8.98' | Found eye-bolt, north end, ± 1' east of conc wall, ± 4' above ground |
| 1866 | 212 | 9.51' | Found eye-bolt, south end, near wood fence, ± 4' above ground |
| 1862 | 213 | 8.20' | Top of conc wall, south end of low wall |
| 1856 | 214 | 8.83' | Found eye-bolt, south end, near conc wall corner, ± 2' above ground |
| 1840 | 215 | 7.74' | Found eye-bolt, south end, in conc near conc wall |
| 1836 | 216 | 8.81' | Found eye-bolt, north end, near conc wall, ± 1' above ground |
| 1828 | 217 | 8.49' | Found eye-bolt, north end, near conc wall corner |
| 1816 | 218 | 8.61' | Found eye-bolt, south end, near conc wall, ± 3.3' above ground |
|  | 219 | 9.24' | FS conc of pipe support, NRD lot north of #1816 |
| 1804 | 220 | 8.75' | Set eye-bolt, south end, west of conc wall |
| 1800 | 221 | 8.36' | Found eye-bolt, north end, ± 2' east of 7' CMP riser, ± 2.4' above ground |
| 1800 | 222 | 7.80' | OG levee |
| 1796 | 223 | 7.95' | Found eye-bolt, north end, ± 2' north of conc wall, ± 1.5' above ground |
| 1792 | 224 | 8.04' | Found eye-bolt, south end, ± 3' north of wood dock, ± 3.5' above ground |
| 1790 | 225 | 8.77' | Found eye-bolt, north end, near wood fence end, ± 2' above ground |
| 1786 | 226 | 8.49' | Found eye-bolt, north end, at end of metal support wall ± 1.2' above ground |
| 1776 | 227 | 9.20' | Found eye-bolt, south end, ± 2' west of guy wire |
| 1772 | 228 | 9.37' | Top of conc wall, north end of wall at corner |
| 1764 | 229 | 8.37' | Found eye-bolt, north end, ± 2' south of conc wall end |
| 1752 | 230 | 9.12' | Found eye-bolt, south end, near chain link fence, ± 2' above ground |
| 1748 | 231 | 8.79' | Found eye-bolt, north end, ± 3' south of metal fence end, ± 2' above ground |
| 1736 | 232 | 9.27' | Top conc wall, south end of wall |
| 1732 | 233 | 9.04' | Top of metal support wall, north end |
| HOUSE NO | POINT NO | ELEV | DESCRIPTION |
| 1720 | 234 | 9.04' | Found eye-bolt, south end, at chain link gate, ± 2.3' above ground |
| 1716 | 235 | 8.97' | Found eye-bolt, north end, north of apple tree, ± 2.5' above ground |
| 1708 | 236 | 9.15' | Top brick wall, north end of wall |
| 1700 | 237 | 8.02' | Top of conc wall, north side of lot, west end of wall |
| 1696 | 238 | 9.08' | Found eye-bolt, north end, near conc wall, ± 3' above ground |
| 1684 | 239 | 8.81' | Found eye-bolt, south end, near wood fence, ± 1.6' above ground |
| 1672 | 240 | 8.57' | Found eye-bolt, north end, ± 1' east of brick pillar, ± 1.7' above ground |
| 1664 | 241 | 8.13' | Found bolt in top of wood wall, north end |
| 1652 | 242 | 8.06' | Found eye-bolt, south end, east of palm tree, ± 2' above ground |
| 1648 | 243 | 9.04' | Found eye-bolt, north end, near conc wall, ± 2.7' above ground |
| 1640 | 244 | 6.99' | Set eye-bolt, north end, ± 1' south of conc wall, north of palm tree |
| 1632 | 245 | 9.09' | Found eye-bolt, north end, at conc wall near corner, ± 1.2' above ground |
| 1620 | 246 | 9.08' | Set eye-bolt, south end, at wire fence |
| 1616 | 247 | 9.49' | Found eye-bolt, north end, ± 2' south of wood wall, ± 2' above ground |
| 1606 | 248 | 8.76' | Top of conc wall at north end |
| 1598 | 249 | 9.17' | Found eye-bolt, south end, near wood wall |
| 1592 | 250 | 8.84' | Set eye-bolt, south end, ± 2' east of conc wall |
| 1584 | 251 | 9.98' | Top of conc wall corner at south end |
| 1580 | 252 | 8.33' | Found eye-bolt, north end, near wood fence |
| 1568 | 253 | 9.37' | Found eye-bolt, south end, near wood fence end, ± 2.6' above ground |
| 1564 | 254 | 9.18' | Found eye-bolt, north end, near conc wall |
| 1556 | 255 | 9.24' | Found eye-bolt, north end, ± 1' south of wood fence, ± 2' above ground |
| 1548 | 256 | 8.25' | Top of conc wall corner at north end of wall |
| 1536 | 257 | 9.23' | Found eye-bolt, south end, ± 1' south of wood retaining wall, ± 3' above ground |
| 1528 | 258 | 9.19' | Found eye-bolt, south end, near conc wall corner |
| 1524 | 259 | 9.34' | Found eye-bolt, north end, ± 3' south of wire fence, ± 1' above ground |
| 1512 | 260 | 9.21' | Found eye-bolt, south end, ± 1' north of wood fence, ± 1' above ground |
| 1500 | 261 | 9.02' | Found eye-bolt, south end |
| 1498 & 1496 | 262 | 8.95' | Set eye-bolt in open area |
| 1460 | 263 | 6.85' | Set eye-bolt, north end, near wood fence end |
| 1428 | 264 | 6.79' | Set eye-bolt, south end, ± 4' east of wood fence end |
| 1428 | 265 | 6.50' | Set eye-bolt, north end, near wood fence end |
| 1412 | 266 | 7.40' | FS wood deck at southeast corner |
| 1400 | 267 | 6.88' | Set eye-bolt, south end, ± 6' north of wood fence |
| 1400 | 268 | 7.06' | Set eye-bolt, north end, near wood fence |
| HOUSE NO | POINT NO | ELEV | DESCRIPTION |
| 1364 | 269 | 6.13' | Set eye-bolt, south end, ± 1' west of conc wall |
| 1360 | 270 | 7.83' | Set eye-bolt, south end, near conc wall |
| 1362 | 271 | 8.46' | Top of conc wall, north end at wall intersection |
| 1354 | 272 | 7.97' | Set eye-bolt, near northeast corner of house |
|  | 273 | 7.96' | FS wood deck |
| 1350 | 274 | 6.89' | Set eye-bolt, north end, ± 3' west of wood fence. Mailbox labeled #1348 |
| 1332 | 275 | 9.48' | Top of conc wall at south end |
| 1331 | 276 | 8.13' | Set eye-bolt, south end, ± 4' north of conc wall |
| 1331 | 277 | 8.24' | Set eye-bolt, north end, ± 15' south of acacia tree |
| 1300 | 278 | 7.86' | Set eye-bolt, south end, ± 10' east of southeast shed corner |
| 1300 | 279 | 8.99' | Set eye-bolt, top of levee, ± 30' north of lot #1300 |
| 1240 | 280 | 6.90' | Set eye-bolt, north end, ± 3' south of wood fence |
| 1222 | 281 | 8.13' | Top of conc wall, east end, ± 30' east of southeast house corner |
| Benchmark: City of Napa # NB-29, Elevation 43.53'. NGVD 1929 |  |  |  |